/**********************************************************************

* TDRP params for Dsr2Vol

**********************************************************************/

//======================================================================

//

// Dsr2Vol program.

//

// Dsr2Volreads an input FMQ containing radar data, and writes it to a

//   file in MDV format. Grid remapping and spatial interpolation are

//   optional.

//

//======================================================================

//======================================================================

//

// DEBUGGING AND PROCESS CONTROL.

//

//======================================================================

///////////// debug ///////////////////////////////////

//

// Debug option.

// If set, debug messages will be printed appropriately.

//

// Type: enum

// Options:

//     DEBUG_OFF

//     DEBUG_NORM

//     DEBUG_VERBOSE

//     DEBUG_EXTRA

//

debug = DEBUG_VERBOSE;

///////////// instance ////////////////////////////////

//

// Process instance.

// Used for registration with procmap.

// Type: string

//

instance = "Test";

//======================================================================

//

// DATA INPUT.

//

//======================================================================

///////////// input_fmq_url ///////////////////////////

//

// Input URL for DsRadar data via FMQ.

// Type: string

//

input_fmq_url = "fmqp:://localhost::./fmq/fmq.dsRadar";

///////////// seek_to_end_of_input ////////////////////

//

// Option to seek to the end of the input FMQ.

// If TRUE, the program will seek to the end of the fmq and only read in

//   new data. If FALSE, it will start reading from the beginning of the

//   FMQ.

// Type: boolean

//

seek_to_end_of_input = TRUE;

///////////// end_of_vol_decision /////////////////////

//

// Decision type for end-of-volume.

// If END_OF_VOL_FLAG, the end-of-volume flag in the data will be used

//   to trigger the end of volume. If CHANGE_IN_VOL_NUM, and end of volume

//   will be assumed when the volume number changes from one beam to the

//   next. If LAST_TILT_IN_VOL, the end of the tilt number given by

//   'last_tilt_in_vol' will be used  to trigger the end-of-volume. If

//   AUTOMATIC, this program will determine the end-of-volume condition by

//   using the antenna-angle information.

//

// Type: enum

// Options:

//     END_OF_VOL_FLAG

//     CHANGE_IN_VOL_NUM

//     LAST_TILT_IN_VOL

//     AUTOMATIC

//

end_of_vol_decision = END_OF_VOL_FLAG;

///////////// last_tilt_in_vol ////////////////////////

//

// The tilt number used to end the volume.

// Only applies if 'end_of_vol_decision' is set to LAST_TILT_IN_VOL.

// Type: int

//

last_tilt_in_vol = 0;

///////////// nbeams_overlap_per_vol //////////////////

//

// The number of beams copied from the previous vol.

// If non-zero, this number of beams is copied from the previous volume

//   into the current one, at the start of the current volume. This is

//   useful if the exact timing of the end-of-vol condition is in

//   question, so that the data around the end-of-vol can be used in both

//   volumes. If you set end_of_vol_decision to AUTOMATIC, it is

//   recommended that you set this value to between 20 and 50, because the

//   end-of-vol decision may be a few beams late.

// Type: int

//

nbeams_overlap_per_vol = 0;

///////////// min_beams_in_vol ////////////////////////

//

// The minimum number of beams in a volume.

// This filters out small unwanted bits of data between genuine volumes.

// Type: int

//

min_beams_in_vol = 1;

///////////// max_beams_in_vol ////////////////////////

//

// The maximum number of beams in a volume.

// This prevents memory from filling up if no end-of-volume is found.

// Type: int

//

max_beams_in_vol = 20000;

///////////// use_input_scan_mode /////////////////////

//

// Option to use the scan_mode in the input data.

// The scan_mode indicates whether the scan is of type SURVEILLANCE/PPI

//   RHI. The scan_mode is optionally included in the beam data. If

//   available, it makes sense to use the scan mode to determine whether

//   to save the data as a SURVEILLANCE/PPI or RHI. Dsr2Vol treats

//   SURVEILLANCE and PPI data in the same manner. However, RHIs are saved

//   in a different way. If the input scan_mode is NOT used, Dsr2Vol uses

//   the antenna angle changes to determine whether the scan is in PPI or

//   RHI mode.

// Type: boolean

//

use_input_scan_mode = FALSE;

///////////// use_input_tilt_numbers //////////////////

//

// Option to use tilt numbers instead of the elevation histogram.

// If TRUE, the tilt numbers in the beam data will be used directly to

//   group the beams into tilts. If FALSE, the antenna angles will be

//   analysed using a histogram technique to determine the tilt numbers.

// Type: boolean

//

use_input_tilt_numbers = FALSE;

//======================================================================

//

// COMPUTING THE ELEVATION HISTOGRAM.

//

// The program needs to determine the elevation angles used in the scan

//   strategy. It does this by creating a histgram of elevation angles,

//   and looking for peaks in this histogram.

//

//======================================================================

///////////// use_target_elev /////////////////////////

//

// Option to use target elevation angles instead of actual.

// If TRUE, the beam elevation will be set to the target elevation

//   angle, and the actual elevation angle will be ignored.

// Type: boolean

//

use_target_elev = FALSE;

///////////// elev_hist_resolution ////////////////////

//

// Resolution of elevation histogram (deg).

// The elevation data is binned at this resolution. If the scan strategy

//   has elevations very close together you may need to decrease this.

// Type: double

//

elev_hist_resolution = 0.1;

///////////// elev_hist_search_width //////////////////

//

// The width of the search in looking for peaks in the histogram.

// When looking for peaks, the program searches by this number of bins

//   on either side of the search bin. For example, if the

//   elev_hist_resolution is 0.1 and the elev_hist_search_width is 3, the

//   program will search 3 bins, or 0.3 degrees, on either side of the

//   search bin. It looks for a peak with values equal to or below the

//   peak in the adjacent bins and less than the peak in bins further out.

// Minimum val: 1

// Type: int

//

elev_hist_search_width = 3;

///////////// elev_hist_start /////////////////////////

//

// Histogram start value (deg).

// The elev value for the lowest bin in the histogram.

// Type: double

//

elev_hist_start = -10;

///////////// elev_hist_end ///////////////////////////

//

// Histogram end value (deg).

// The elev value for the highest bin in the histogram.

// Type: double

//

elev_hist_end = 90;

///////////// specify_elev_delta //////////////////////

//

// Option to specify the delta elevation instead of using the elevation

//   histogram.

// This is useful if you have RHI data as input but want to store the

//   output as PPIs. Specify the delta elevation.

// Type: boolean

//

specify_elev_delta = FALSE;

///////////// elev_delta //////////////////////////////

//

// The delta elevation to be used (deg).

// See 'specify_elev_delta'.

// Type: double

//

elev_delta = 1;

//======================================================================

//

// CHECKING TILT DATA.

//

// Checking whether tilts should be included.

//

//======================================================================

///////////// check_min_beams_in_tilt /////////////////

//

// Option to check the number of beams in a tilt.

// If TRUE, a tilt is only considered valid if the number of beams

//   exceeds 'min_beams_in_tilt'.

// Type: boolean

//

check_min_beams_in_tilt = TRUE;

///////////// min_beams_in_tilt ///////////////////////

//

// The min number of beams for a tilt to be valid.

// See 'check_min_beams_in_tilt'.

// Type: int

//

min_beams_in_tilt = 180;

///////////// check_min_fraction_in_tilt //////////////

//

// Option to check the fraction of beams in a tilt.

// If TRUE, a tilt is only considered valid if the number of beams in

//   the tilt, expressed as a fraction of the max number of beams in any

//   tilt, exceeds 'min_fraction_in_tilt'.

// Type: boolean

//

check_min_fraction_in_tilt = TRUE;

///////////// min_fraction_in_tilt ////////////////////

//

// The min fraction of max beams for a tilt to be valid.

// See 'check_min_fraction_in_tilt'.

// Type: double

//

min_fraction_in_tilt = 0.5;

//======================================================================

//

// FILTERING USING SPECIFIED FIELD - NEW.

//

// You have the option of filtering the output data based on the value

//   of a specified field. If the field at a gate does not fall within the

//   specified range, all of the output fields will be set to missing.

// See also 'check_sn' for the older method of filtering on

//   Signal-to-noise computed from DBZ.

//

//======================================================================

///////////// filter_output_using_thresholds //////////

//

// Option to filter the output using set thresholds.

// If TRUE, the thresholding fields will be examined to see if it is

//   within the desired range. Examples are SNR (Signal-to-noise) and NCP

//   (normalized coherent power). If the specified field at a gate falls

//   outside the specified range, all output fields will be set to missing

//   for that gate.

// Type: boolean

//

filter_output_using_thresholds = FALSE;

///////////// threshold_field_name ////////////////////

//

// Name of field for thresholding.

// This is the DSR (input) field name for the thresholding field. If

//   this field is available, it is used for thresholding. If not,

//   thresholding will not be performed.

// Type: string

//

threshold_field_name = "NCP";

///////////// threshold_min_value /////////////////////

//

// Minimum threshold - see 'filter_output_using_thresholds'.

// The specified field at a gate must exceed this value for the gate to

//   be accepted.

// Type: double

//

threshold_min_value = 0.3;

///////////// threshold_max_value /////////////////////

//

// Maximum threshold - see 'filter_output_using_thresholds'.

// The specified field at a gate must be less than this value for the

//   gate to be accepted.

// Type: double

//

threshold_max_value = 1.1;

//======================================================================

//

// NOISE FILTERING USING SNR FROM DBZ - OLD.

//

// You have the option of filtering out noise values based on the

//   signal/noise ratio. If check_sn is true, the program filters out all

//   gates with signal/noise values below the threshold. Additionally,

//   isolated spikes of reflectivity with high signal/noise may be

//   filtered out by requiring that a valid weather echo will have a

//   radial dimension which exceeds a given number of gates - see

//   'sn_min_valid_run'.

//

//======================================================================

///////////// check_sn ////////////////////////////////

//

// Option to check the signal/noise values.

// If TRUE, the signal/noise value will be checked at each gate. If this

//   exceeds the parameter 'sn_threshold', it will be conditionally

//   accepted. See also sn_min_valid_run for additional filtering.

// Type: boolean

//

check_sn = FALSE;

///////////// sn_threshold ////////////////////////////

//

// Signal/noise threshold - for noise_removal.

// See 'check_sn'.

// Type: double

//

sn_threshold = 0;

///////////// sn_min_valid_run ////////////////////////

//

// Minimum valid run of reflectivity above the noise.

// Only active if set to 2 or greater. A second check is made to remove

//   noise spikes. Looking along the radial the number of contiguous gates

//   (a 'run') with sn values above the threshold is computed. For the

//   gates in this run to be accepted the length of the run must exceed

//   sn_min_valid_run.

// Type: int

//

sn_min_valid_run = 0;

///////////// noise_dbz_at_100km //////////////////////

//

// The noise represented as dBZ at 100km range.

// This is used for computing the signal/noise value from the DBZ field.

// Type: double

//

noise_dbz_at_100km = 0;

//======================================================================

//

// FILLING IN MISSING BEAMS.

//

//======================================================================

///////////// bridge_missing_in_azimuth ///////////////

//

// Option to bridge over missing beams in azimuth.

// If true, missing beams in azimuth will be filled in by interpolating

//   from the adjacent two beams. Only a single missing beam will be

//   filled in. If more than one is missing, they will not be filled in.

//   Azimuth filling is done first, before elevation filling.

// Type: boolean

//

bridge_missing_in_azimuth = FALSE;

///////////// bridge_missing_in_elevation /////////////

//

// Option to bridge over missing beams in elevation.

// If true, missing beams in elevation will be filled in by

//   interpolating from the adjacent two beams. Only a single missing beam

//   will be filled in. If more than one is missing, they will not be

//   filled in. The elevation interpolation is done after the asimuth

//   interpolation is complete.

// Type: boolean

//

bridge_missing_in_elevation = FALSE;

//======================================================================

//

// CHECK FOR MOVING ANTENNA.

//

//======================================================================

///////////// check_antenna_moving ////////////////////

//

// Option to check that the antenna is moving.

// If true, beams will only be stored if the antenna is moving. If the

//   antenna stops, beams will be ignored.

// Type: boolean

//

check_antenna_moving = FALSE;

///////////// min_angle_change ////////////////////////

//

// Minimun angle change beam-to-beam (deg).

// For the antenna to be considered moving. the antenna much move by

//   this amount from one beam to the next. The angle check is made in

//   both azimuth and elevation.

// Type: double

//

min_angle_change = 0.05;

//======================================================================

//

// REMOVING TEST PULSE.

//

//======================================================================

///////////// remove_test_pulse ///////////////////////

//

// Option to remove the test pulse data.

// If true, the test pulse data will be removed from the end of the

//   beam. The test pulse data generally lies in the last few gates in the

//   beam. If true, the number of gates will be reduced by

//   'ngates_test_pulse'.

// Type: boolean

//

remove_test_pulse = FALSE;

///////////// ngates_test_pulse ///////////////////////

//

// Number of gates to remove to get rid of test pulse.

// See 'remove_test_pulse'.

// Type: int

//

ngates_test_pulse = 20;

//======================================================================

//

// RADAR PARAMETERS.

//

//======================================================================

///////////// delta_az ////////////////////////////////

//

// Target delta azimuth (deg).

// The target spacing of the data beams in azimuth. NOTE: the lookup

//   table is computed assuming that there is an exact number of beams per

//   45-degree sector. If delta_az does not satisfy this requirement, it

//   will be adjusted upwards to the next suitable value.

// Type: double

//

delta_az = 1;

///////////// az_correction ///////////////////////////

//

// Correction to angular azimuth values (deg).

// This correction is applied to the azimuth angles before computing the

//   azimuth position.

// Type: double

//

az_correction = 0;

///////////// override_radar_location /////////////////

//

// Option to override radar location.

// If TRUE, the program will use location specified in the

//   'radar_location' parameter.

// Type: boolean

//

override_radar_location = FALSE;

///////////// radar_location //////////////////////////

//

// Radar location if override is set true.

// The radar_location is only used if 'override_radar_location' is set

//   true. Otherwise the information in the input data stream is used.

//   Note that the altitude is in kilometers, not meters.

//

// Type: struct

//   typedef struct {

//      double latitude;

//      double longitude;

//      double altitude;

//   }

//

//

radar_location = {

latitude = 0,

longitude = 0,

altitude = 0

};

///////////// override_beam_width /////////////////////

//

// Option to override radar beam width.

// If TRUE, the program will use beam width specified in the

//   'beam_width' parameter.

// Type: boolean

//

override_beam_width = FALSE;

///////////// beam_width //////////////////////////////

//

// Radar beam width if override is set true.

// The beam width is only used if 'override_beam_width' is set true.

//   Otherwise the information in the input data stream is used.

// Type: double

//

beam_width = 1;

///////////// radar_description ///////////////////////

//

// General description of radar.

// Used for data_set_info in MDV file.

// Type: string

//

radar_description = "not-set";

//======================================================================

//

// FILTER DATA BASED ON GEOMETRY or PRF.

//

//======================================================================

///////////// filter_gate_spacing /////////////////////

//

// Option to filter data based on gate spacing.

// If TRUE, the program will use only beam data which matches the

//   'keep_gate_spacing' parameter.

// Type: boolean

//

filter_gate_spacing = FALSE;

///////////// keep_gate_spacing ///////////////////////

//

// Desired radar gate spacing (km).

// The specified gate spacing is only used if 'filter_gate_spacing' is

//   set true. Otherwise all gate spacings in the input data stream are

//   used.

// Type: double

//

keep_gate_spacing = 0.25;

///////////// filter_start_range //////////////////////

//

// Option to filter data based on start range.

// If TRUE, the program will use only beam data which matches the 'keep

//   _start_range' parameter.

// Type: boolean

//

filter_start_range = FALSE;

///////////// keep_start_range ////////////////////////

//

// Desired radar start range (km).

// The specified start range is only used if 'filter_start_range' is set

//   true. Otherwise all start ranges in the input data stream are used.

// Type: double

//

keep_start_range = 0;

///////////// filter_prf //////////////////////////////

//

// Option to filter data based on PRF.

// If TRUE, the program will use only beam data with PRFs between the

//   given limits.

// Type: boolean

//

filter_prf = FALSE;

///////////// min_prf /////////////////////////////////

//

// Minimum acceptable PRF (/s).

// See 'filter_prf'.

// Type: double

//

min_prf = 100;

///////////// max_prf /////////////////////////////////

//

// Maximum acceptable PRF (/s).

// See 'filter_prf'.

// Type: double

//

max_prf = 2000;

///////////// filter_elev /////////////////////////////

//

// Option to filter data based on elevation angle.

// If TRUE, the program will use only beam data with elevations between

//   the given limits.

// Type: boolean

//

filter_elev = FALSE;

///////////// min_elev ////////////////////////////////

//

// Minimum valid elevation for beams (deg).

// If the elevation is below this value, the beam is ignored.

// Type: double

//

min_elev = -10;

///////////// max_elev ////////////////////////////////

//

// Maximum valid elevation for beams (deg).

// If the elevation is above this value, the beam is ignored.

// Type: double

//

max_elev = 90;

///////////// filter_antenna_transitions //////////////

//

// Option to filter data when the transition flag is set.

// If TRUE, the program will filter out beams which are computed when

//   the antenna is moving from one tilt to the next, or one volume to the

//   next.

// Type: boolean

//

filter_antenna_transitions = FALSE;

//======================================================================

//

// INTERPOLATION.

//

//======================================================================

///////////// min_nvalid_for_interp ///////////////////

//

// Minimum number of valid data points for theinterpolation.

// The program performs an 8-point linear interpolation. This is the

//   number of valid data points, out of the possible 8, which must be

//   present for interpolation to proceed. A high number will exclude

//   marginal points. A low number will include marginal points.

// Minimum val: 1

// Maximum val: 8

// Type: int

//

min_nvalid_for_interp = 5;

//======================================================================

//

// OUTPUT FIELDS.

//

//======================================================================

///////////// output_compression //////////////////////

//

// Output compression options.

// The data may be optionally compressed for output. BZIP is generally

//   the slowest but most compact. ZLIB uses the gzip compression

//   algorithm, which is faster than BZIP, and for some data more compact

//   as well. LZO is faster than BZIP and ZLIB, but not as compact. RLE is

//   simple run-length encoding compression, which is the fastest and

//   least compact.

//

// Type: enum

// Options:

//     NO_COMPRESSION

//     RLE_COMPRESSION

//     LZO_COMPRESSION

//     ZLIB_COMPRESSION

//     BZIP_COMPRESSION

//     GZIP_COMPRESSION

//

output_compression = ZLIB_COMPRESSION;

///////////// output_fields ///////////////////////////

//

// Array of output fields.

// The program will only output these fields. The dsr_name is the field

//   name in the dsr data. If the output_name is an empty string, the

//   output name will be set to the dsr_name. If output_name is non-empty,

//   this will be used. Likewise for the units. Set the transform to dB,

//   dBZ or none. Indicate the dBZ field by setting is_dbz to true. Set

//   interp_db_as_power if you want to compute power from db before

//   interpolating. Set is_vel for velocity field so that interpolation

//   can take nyquist folding into account. If allow_interp is set to

//   false, interpolation is not permitted on this field, and

//   nearest-neighbor will be used instead. Set the output encoding type

//   to select the resolution of the output data.

//

// Type: struct

//   typedef struct {

//      string dsr_name;

//      string output_name;

//      string output_units;

//      string transform;

//      boolean is_dbz;

//      boolean interp_db_as_power;

//      boolean is_vel;

//      boolean allow_interp;

//      encoding_t encoding;

//        Options:

//          ENCODING_INT8

//          ENCODING_INT16

//          ENCODING_FLOAT32

//   }

//

// 1D array - variable length.

//

output_fields = {

{

dsr_name = "DBZ",

output_name = "DBZ",

output_units = "dBZ",

transform = "none",

is_dbz = TRUE,

interp_db_as_power = FALSE,

is_vel = FALSE,

allow_interp = TRUE,

encoding = ENCODING_INT8

}

,

{

dsr_name = "VEL",

output_name = "VEL",

output_units = "m/s",

transform = "none",

is_dbz = FALSE,

interp_db_as_power = FALSE,

is_vel = TRUE,

allow_interp = TRUE,

encoding = ENCODING_INT8

}

};

///////////// output_coverage_field ///////////////////

//

// Option to output a field depicting radar coverage.

// If true, and extra field, with the name 'Coverage', is included in

//   the output. This is a simple flag field, with a 1 indicating that the

//   radar covers that pixel, and a 0 indicating that is does not.

// Type: boolean

//

output_coverage_field = FALSE;

///////////// override_missing_dbz_for_interp /////////

//

// Option to set dBZ values if they are missing.

// If true, missing dBZ values will be replaced by a low dbz value

//   before interpolation takes place. If this is not done, non-missing

//   dBZ values tend to be propagated into the missing areas, patricularly

//   in the vertical dimension at long range where the vertical spacing

//   between the beams is large. This leads to a ballooning of

//   reflectivity from low heights.

// Type: boolean

//

override_missing_dbz_for_interp = TRUE;

///////////// override_nyquist ////////////////////////

//

// Option to override nyquist velocity in incoming data.

// If true, the nyquist_velocity parameter is used to specify the

//   nyquist. If false, the nyquist is computed from the incoming radar

//   data stream. The nyquist is used for velocity interpolation, to

//   ensure that folded values are treated correctlty.

// Type: boolean

//

override_nyquist = FALSE;

///////////// nyquist_velocity ////////////////////////

//

// Specify nyquist velocity (m/s).

// See 'override_nyquist'.

// Type: double

//

nyquist_velocity = 25;

//======================================================================

//

// OUTPUT FILES.

//

//======================================================================

///////////// output_cart_files ///////////////////////

//

// Option to output files with Cartesian geometry.

// If true, the parameters in the cart_files array are used. The grid

//   origin is at the radar. nxy is the number of grid points in x and y.

//   dxy is the grid resolution in x and y. Therefore the grid is a square

//   in x,y. nz is the number of CAPPI heights. minz is the height of the

//   lowest CAPPI and dz is the height between successive CAPPIs. If

//   interpolate is true, an 8-point linear interpolation grid

//   transformation is performed. If interpolate is false, a

//   nearest-neighbor transformation is performed. The MDV files are

//   stored at mdv_url. The max_range parameter (km) can be used to limit

//   the useful data range. Input data beyond this range is ignored. Set

//   to a negative value to disable it, in which case no limit is placed

//   on range.

// Type: boolean

//

output_cart_files = TRUE;

///////////// cart_files //////////////////////////////

//

// Details for files in cart geometry.

// Only active if 'output_cart_files' is true.

//

// Type: struct

//   typedef struct {

//      int nxy;

//      double dxy;

//      int nz;

//      double minz;

//      double dz;

//      boolean interpolate;

//      string mdv_url;

//      double max_range;

//   }

//

// 1D array - variable length.

//

cart_files = {

{

nxy = 400,

dxy = 1,

nz = 20,

minz = 0.5,

dz = 1,

interpolate = TRUE,

mdv_url = "/polcast2/titan5/projDir/data/mdv/radarCart",

max_range = -1

}

};

///////////// output_ppi_files ////////////////////////

//

// Option to output files with PPI geometry.

// If true, the parameters in the ppi_files array are used. The grid

//   origin is at the radar. nxy is the number of grid points in x and y.

//   dxy is the grid resolution in x and y. Therefore the grid is a square

//   in x,y. min_elev is the lowest elevation angle to be stored, in

//   degrees. Similarly for for max_elev. If interpolate is true, an

//   8-point linear interpolation grid transformation is performed. If

//   interpolate is false, a nearest-neighbor transformation is performed.

//   The MDV files are stored at mdv_url. All gates with heights below

//   min_ht or above max_ht (in km) will be set to missing. The max_range

//   parameter (km) can be used to limit the useful data range. Input data

//   beyond this range is ignored. Set to a negative value to disable it,

//   in which case no limit is placed on range.

// Type: boolean

//

output_ppi_files = FALSE;

///////////// ppi_files ///////////////////////////////

//

// Details for files in ppi geometry.

// Only active if 'output_ppi_files' is true.

//

// Type: struct

//   typedef struct {

//      int nxy;

//      double dxy;

//      double min_elev;

//      double max_elev;

//      boolean interpolate;

//      string mdv_url;

//      double min_ht;

//      double max_ht;

//      double max_range;

//   }

//

// 1D array - variable length.

//

ppi_files = {

{

nxy = 400,

dxy = 1,

min_elev = 0,

max_elev = 90,

interpolate = FALSE,

mdv_url = "/polcast2/titan5/projDir/data/mdv/radarCart",

min_ht = 0,

max_ht = 50,

max_range = -1

}

};

///////////// output_polar_files //////////////////////

//

// Option to output files with Polar radar geometry.

// If true, the parameters in the polar_files array are used. max_range

//   is the maximum range to be stored, in km. min_elev is the lowest

//   elevation angle to be stored, in degrees. Similarly for for max_elev.

//   If interpolate is true, an 8-point linear interpolation grid

//   transformation is performed. If interpolate is false, a

//   nearest-neighbor transformation is performed. The MDV files are

//   stored at mdv_url.

// Type: boolean

//

output_polar_files = TRUE;

///////////// polar_files /////////////////////////////

//

// Details for files in polar geometry.

// Only active if 'output_polar_files' is true.

//

// Type: struct

//   typedef struct {

//      double max_range;

//      double min_elev;

//      double max_elev;

//      string mdv_url;

//   }

//

// 1D array - variable length.

//

polar_files = {

{

max_range = 300,

min_elev = 0,

max_elev = 90,

mdv_url = "/polcast2/titan5/projDir/data/mdv/radarPolar"

}

};

///////////// trim_polar_sectors //////////////////////

//

// Option to store only the sector in which there is data.

// This only applies to polar files. In the MDV file a sector may be

//   stored as a full 360 or with only the beams in which there is data -

//   the rest of the azimuths will be filled with missing values. If this

//   is set TRUE, only the sector in which data is present will be stored.

// Type: boolean

//

trim_polar_sectors = FALSE;

//======================================================================

//

// SEPARATING SECTOR VOLUMES FROM SURVEILLANCE VOLUMES.

//

//======================================================================

///////////// separate_sector_files ///////////////////

//

// Option to separate the sector files from surveillance files.

// Sometimes a scan strategy will switch between sector scans and

//   surveillance scans. In these cases, it can be useful to separate the

//   sectors by storing them in different directories.

// If you set this option to true, the directory path for sectors will

//   be determined by appending the 'sector_subdirectory' parameter to the

//   paths of cart, ppi or polar files. Similarly, for surveillance scans,

//   the parameter 'surveillance_subdirectory' will be appended to the

//   paths.

// If the parameter 'use_input_scan_mode' is true, the scan mode will be

//   used to determine which scans are sectors. If use_input_scan_mode is

//   false, the number of beams will be computed as a fraction of the

//   maximum number possible. If this fraction is greater than the

//   parameter 'min_fraction_for_surveillance', the scan will be assumed

//   to be a surveillance scan. Otherwise it will be assumed to be a

//   sector scan.

// Type: boolean

//

separate_sector_files = FALSE;

///////////// sector_subdirectory /////////////////////

//

// The directory name for sector scan files.

// See 'separate_sector_files'. If a volume is a sector, this will be

//   appended to the path for cart, ppi and polar files.

// Type: string

//

sector_subdirectory = "sector";

///////////// surveillance_subdirectory ///////////////

//

// The directory name for surveillance scan files.

// See 'separate_sector_files'. If a volume is a surveillance scan, this

//   will be appended to the path for cart, ppi and polar files.

// Type: string

//

surveillance_subdirectory = "full";

///////////// min_fraction_for_surveillance ///////////

//

// The min fraction of max beams for a surveillance scan.

// See 'separate_sector_files'. If the fraction of beams in the volume

//   exceeds this min value, the volume will be considered a full

//   revolution. Otherwise it will be considered a sector scan. Only

//   active if 'use_input_scan_mode' is false.

// Type: double

//

min_fraction_for_surveillance = 0.8;

//======================================================================

//

// SEPARATE VERTICALLY-SCANNED VOLUMES.

//

//======================================================================

///////////// separate_vert_files /////////////////////

//

// Option to write vertically-pointing data files to a separate

//   directory.

// This allows you to separate volumes of vertically-pointing data and

//   save them to a separate directory. If the elevation angles in a

//   volume are consistently above a specified value, the volume will be

//   written to the specified directory. This only applies to polar data.

//   If is ignored for cart and ppi output.

// Type: boolean

//

separate_vert_files = FALSE;

///////////// vert_subdirectory ///////////////////////

//

// The directory name for vertically-pointing volumes.

// See 'separate_vert_files'.

// Type: string

//

vert_subdirectory = "vert";

///////////// min_elevation_for_vert_files ////////////

//

// The min elevation angle for vertically-pointing operations (deg).

// This is the elevation threshold for vertically-pointing operations.

//   If the specified fraction of the data has an elevation angle in

//   excess of this angle, the volume will be considered to be from

//   vertically-pointing operations.

// Type: double

//

min_elevation_for_vert_files = 85;

///////////// min_vert_fraction_for_vert_files ////////

//

// The min fraction of data with elevations in excess of the min

//   elevation.

// If the fraction of the data with elevation angles above the specified

//   min_elevation exceeds this fraction,  the volume will be considered

//   to be from vertically-pointing operations.

// Type: double

//

min_vert_fraction_for_vert_files = 0.9;

//======================================================================

//

// RHI MODE - OPTION TO SAVE RHI DATA.

//

// If the program finds RHI data, it performs a histogram analysis to

//   decide which radials are active and stores out the data in the MDV

//   files as RHIs.

//

//======================================================================

///////////// output_rhi_files ////////////////////////

//

// Option to save RHIs if available.

// When creating RHI files, the elevation angles are stored in a regular

//   grid while the azimuths are stored in the vlevels array.

// If the beams are indexed in elevation (i.e. are computed on a regular

//   grid), the RHIs are saved out on that regular grid.

// If the beams are not indexed, the actual RHI elevation angles are

//   resampled onto the regular grid. The oversampling ratio is computed

//   as the number of elevation angles stored divided by the number of

//   elevations measured. The higher the number the more accurate the

//   elevation values. This step is necessary because the measured angles

//   vary from one RHI to another, but all RHIs must be stored with the

//   same elevation angles. Oversampling allows the program to interpolate

//   the measured values onto the stored angles.

// The interpolation flag governs whether the RHI data is interpolated

//   when it is put onto the regular elevation grid. If false, nearest

//   neighbor is used.

// Type: boolean

//

output_rhi_files = FALSE;

///////////// rhi_files ///////////////////////////////

//

// Details for files in RHI geometry.

// Only active if 'output_rhi_files' is true.

//

// Type: struct

//   typedef struct {

//      double oversampling_ratio;

//      boolean interp_in_elevation;

//      string mdv_url;

//   }

//

// 1D array - variable length.

//

rhi_files = {

{

oversampling_ratio = 4,

interp_in_elevation = FALSE,

mdv_url = "./data/mdv/radarRhi"

}

};

///////////// rhi_az_hist_resolution //////////////////

//

// Resolution of azimuth histogram (deg).

// For determining RHI azimuths. The azimuth data is binned at this

//   resolution.

// Type: double

//

rhi_az_hist_resolution = 0.1;

///////////// rhi_az_hist_search_width ////////////////

//

// The width of the search in looking for peaks in the azimuth

//   histogram.

// When looking for peaks in azimuth histogram, the program searches by

//   this number of bins on either side of the search bin. For example, if

//   the az_hist_resolution is 0.1 and the az_hist_search_width is 3, the

//   program will search 3 bins, or 0.3 degrees, on either side of the

//   search bin. It looks for a peak with values equal to or below the

//   peak in the adjacent bins and less than the peak in bins further out.

// Minimum val: 1

// Type: int

//

rhi_az_hist_search_width = 3;

///////////// check_min_beams_in_rhi //////////////////

//

// Option to check the number of beams in an rhi.

// If TRUE, an rhi is only considered valid if the number of beams

//   exceeds 'min_beams_in_rhi'.

// Type: boolean

//

check_min_beams_in_rhi = FALSE;

///////////// min_beams_in_rhi ////////////////////////

//

// The min number of beams for an rhi to be valid.

// See 'check_min_beams_in_rhi'.

// Type: int

//

min_beams_in_rhi = 20;

///////////// check_min_fraction_in_rhi ///////////////

//

// Option to check the fraction of beams in an rhi.

// If TRUE, an rhi is only considered valid if the number of beams in

//   the rhi, expressed as a fraction of the max number of beams in any

//   rhi, exceeds 'min_fraction_in_rhi'.

// Type: boolean

//

check_min_fraction_in_rhi = FALSE;

///////////// min_fraction_in_rhi /////////////////////

//

// The min fraction of max beams for an rhi to be valid.

// See 'check_min_fraction_in_rhi'.

// Type: double

//

min_fraction_in_rhi = 0.3;

//======================================================================

//

// OUTPUT FILE TIME-STAMP.

//

//======================================================================

///////////// auto_mid_time ///////////////////////////

//

// Option to automatically compute the volume mid time.

// If TRUE, the mid_time is the mean of the start and end times of the

//   data in the volume. If FALSE, the mid time is computed as the end

//   time minus 'age_at_end_of_volume'.

// Type: boolean

//

auto_mid_time = FALSE;

///////////// age_at_end_of_volume ////////////////////

//

// Specified age (secs) of the data relative to the data time at the end

//   of the volume.

// Used if 'auto_mid_time' is FALSE.

// Type: int

//

age_at_end_of_volume = 0;

///////////// max_vol_duration ////////////////////////

//

// Maximum volume duration (secs).

// If a volume exceeds this duration it is not saved. This takes care of

//   cases in which the radar data stops in the middle of a volume for

//   some reason. Then the early data in the volume will not match the

//   late data.

// Type: int

//

max_vol_duration = 900;

//======================================================================

//

// OUTPUT DATA SET INFORMATION.

//

//======================================================================

///////////// data_set_info ///////////////////////////

//

// Data set info.

// This is placed in the MDV master header for documentation purposes.

// Type: string

//

data_set_info = "This MDV radar volume file was created by Dsr2Vol.";

///////////// data_set_source /////////////////////////

//

// Data set source details.

// This is placed in the MDV master header for documentation purposes.

// Type: string

//

data_set_source = "Radar data, RAP, NCAR.";

//======================================================================

//

// AUTOMATIC END-OF-VOLUME DETECTION.

//

// These parameters are used if end_of_volume_decision is set to

//   AUTOMATIC. This assumes a simple bottom-up or top-down scan strategy.

//   No attempt will be made to search for RHIs, or other complicattions

//   in the SCAN strategy.

//

//======================================================================

///////////// set_end_of_vol_from_elev_change /////////

//

// Option to set the end of vol flag based on change in elevation angle.

// If true, the program will look for changes in elevation angle to

//   compute the end of volume. Only this test, and the test for prf

//   change, will be used to identify the end of volume condition.

// Type: boolean

//

set_end_of_vol_from_elev_change = FALSE;

///////////// vol_starts_at_bottom ////////////////////

//

// Flag to indicate that elevation angles increase in the volume.

// If the volume starts at the top, i.e. elevations decrease during the

//   volume, set this to FALSE.

// Type: boolean

//

vol_starts_at_bottom = TRUE;

///////////// elev_change_for_end_of_vol //////////////

//

// Change in elevation which triggers and end of volume.

// If the elevation angle changes by this amount, and end of volume flag

//   will be set.

// Minimum val: 0.1

// Type: double

//

elev_change_for_end_of_vol = 1.5;

///////////// set_end_of_vol_on_prf_change ////////////

//

// Option to set the end-of-vol flag when the PRF changes.

// Type: boolean

//

set_end_of_vol_on_prf_change = FALSE;

///////////// nbeams_history //////////////////////////

//

// Number of beams in history list.

// This is the number of beams stored in the history list. The list is

//   used for determining the status of the antenna. The antenna algorithm

//   checks o see if either the elevation angle is stable (PPI mode) or

//   the azimuth angle is stable (RHI mode). The el_accuracy and

//   az_accuracy parameters are used to detect whether el or az are

//   stable.

// Type: int

//

nbeams_history = 16;

///////////// el_accuracy /////////////////////////////

//

// Accuracy of antenna controller in elevation (deg).

// Used to determine if antenna is stationary in elevation, i.e. is it

//   in PPI mode? If the cumulative elevation change during the history

//   period is less than this value, the antenna is assumed to be in PPI

//   mode.

// Type: double

//

el_accuracy = 0.25;

///////////// az_accuracy /////////////////////////////

//

// Accuracy of antenna controller in azimuth (deg).

// Used to determine if antenna is stationary in azimuth, i.e. is it in

//   RHI mode? If the cumulative azimuth change during the history period

//   is less than this value, the antenna is assumed to be in RHI mode.

// Type: double

//

az_accuracy = 0.25;

///////////// min_az_change_ppi ///////////////////////

//

// Min azimuth change for PPI (deg).

// For a valid PPI, the azimuth must change by at least this amount

//   during nbeams_history, while the elevation is not changing.

// Type: double

//

min_az_change_ppi = 4;

///////////// min_el_change_rhi ///////////////////////

//

// Min elevation change for RHI (deg).

// For a valid RHI, the elevation must change by at least this amount

//   during nbeams_history, while the azimuth is not changing.

// Type: double

//

min_el_change_rhi = 4;

///////////// max_az_change_per_tilt //////////////////

//

// Maximum azimuth change per tilt (deg).

// If the tilt elevation has not changed by the time the azimuth has

//   changed by this number of degrees, an end-of-volume condition is

//   triggered. This is used to handle the single-elevation surveillance

//   case in which the antenna scans in PPI at a single elevation angle.

// Type: int

//

max_az_change_per_tilt = 540;

///////////// min_beams_per_ppi_vol ///////////////////

//

// Min number of beams per PPI volume.

// Used to prevent end-of-volume conditions being determined during

//   antenna transition to the start of the next volume.

// Type: int

//

min_beams_per_ppi_vol = 180;

///////////// min_beams_per_rhi_vol ///////////////////

//

// Min number of beams per RHI volume.

// Used to prevent end-of-volume conditions being determined during

//   antenna transition to the start of the next volume.

// Type: int

//

min_beams_per_rhi_vol = 30;

///////////// debug_auto_detection ////////////////////

//

// Flag for debugging auto end-of-vol detection.

// If set, messages will be printed to stderr on how the auto-detection

//   algorithm is making decisions.

// Type: boolean

//

debug_auto_detection = FALSE;