/**********************************************************************

* TDRP params for EsdAcIngest

**********************************************************************/

//======================================================================

//

// EsdAcIngest reads aircraft data from an ASCII string and stores it in

//   Spdb.

//

//======================================================================

//======================================================================

//

// DEBUGGING AND PROCESS CONTROL.

//

//======================================================================

///////////// debug ///////////////////////////////////

//

// Debug option.

// If set, debug messages will be printed appropriately.

//

// Type: enum

// Options:

//     DEBUG_OFF

//     DEBUG_NORM

//     DEBUG_VERBOSE

//

debug = DEBUG_VERBOSE;

///////////// instance ////////////////////////////////

//

// Process instance.

// Used for registration with procmap.

// Type: string

//

instance = "N373";

///////////// mode ////////////////////////////////////

//

// Operating mode.

// In REALTIME mode, the aircraft data is read in from the serial

//   connection. In ARCHIVE mode the data is read in from files, using the

//   -f command line option. In TCP mode, reads data from a TCP server. In

//   TEST mode, aircraft data is generated for testing purposes.

//

// Type: enum

// Options:

//     REALTIME

//     ARCHIVE

//     TCP

//     TEST

//

mode = ARCHIVE;

//======================================================================

//

// SERIAL DATA INPUT.

//

// The original system used a fixed-length preformatted string. The

//   newer system uses a comma-delimited string. The program automatically

//   determines which type of input string is being used.

//

//======================================================================

///////////// input_format ////////////////////////////

//

// Data input format.

// 	AUTOMATIC_FORMAT - The program automatically determine the data

//   format. This can only be done if the data is in one of the following

//   formats:

// 		COMMA_DELIMITED_FORMAT

// 		LONG_FIXED_STRING_FORMAT

// 		WMI_LONG_STRING_FORMAT

// 		WMI_STRING_FORMAT

// 	COMMA_DELIMITED_FORMAT - The fields in the data are delimited by

//   commas. The line begins with the following fields:

// 		call sign

// 		year

// 		month

// 		day

// 		hour

// 		minute

// 		second

// 		latitude

// 		longitude

// 		altitude

// These fields are followed by the optional fields specified in the

//   optional_field_names parameter.

// 	LONG_FIXED_STRING_FORMAT - The data is in a white-space delimited

//   format. The fields appear in the following order:

// 		hour

// 		minute

// 		second

// 		pressure

// 		temperature

// 		dew point

// 		wind speed

// 		wind direction

// 		longitude

// 		latitude

// 		altitude

// 		king lwc

// 		fssp conc

// 		rosemount volts

// Since the call sign isn't specified in the data, the first call sign

//   in the callsigns parameter is used as the call sign for this data. If

//   the callsigns parameter is empty, the resulting AC position record

//   won't have a call sign associated with it.

// 	WMI_LONG_STRING_FORMAT - The data is in the WMI long string format.

//   In this format, the data is all in a long line with no delimiters

//   with the following fields:

// 		call sign

// 		latitude * 10000

// 		longitude * 10000

// 		tas

// 		GPS altitude * 100

// 		pressure altitude

// 		tdry * 10

// 		lwjw * 100

// 		dew point * 10

// 		fssp

// 		burn in place

// 		number of burn in place

// 		ejectable

// 		number of ejectable

// 		left burn

// 		right burn

// 		ice hopper

// 		GPS time string

// 		GPS var

// 		GPS error

// 		GPS warn

// 	WMI_STRING_FORMAT - The data is received in the old WMI string

//   format. In this format, the data is all in a long line with no

//   delimiters with the following fields:

// 		call sign

// 		latitude

// 		longitude

// 		tas

// 		GPS altitude

// 		pressure altitude

// 		tdry

// 		lwjw

// 		???

// 		number of burn in place

// 		number of ejectable

// 		right burn

// 		left burn

// 		???

// 		burn in place

// 		ejectable

// 		GPS hour

// 		GPS minute

// 		GPS second

// 		GPS var

// 		GPS error

// 		airnav warn

// 	SEA_M300_FORMAT - This data is in white-space delimited files and

//   contains the following fields:

// 		time in hh:mm:ss format

// 		date in m/dd/yyyy format

// 		latitude

// 		longitude

// 		altitude in feet

// 		altitude in meters

// 		temperature

// 		relative humidity

// 		CN concentration

// 		CCN concentration

// 		CCNSS

// 		CCN flag

// 		PCASP

// 		P1 - P15

// 		SPPRng

// 		SPP

// 		SPPLWC

// 		KPLWC

// 		mean dia

// 		MVD

// 		EffRad

// 		standard deviation

// 		disp

// 		2DCshad

// 		2DPshad

// 		S1 - S20.

//

// Type: enum

// Options:

//     AUTOMATIC_FORMAT

//     COMMA_DELIMITED_FORMAT

//     LONG_FIXED_STRING_FORMAT

//     WMI_LONG_STRING_FORMAT

//     WMI_STRING_FORMAT

//     SEA_M300_FORMAT

//

input_format = COMMA_DELIMITED_FORMAT;

///////////// optional_field_names ////////////////////

//

// Array of variable fields following fixed fields.

// Used for comma-delimited format only. The comma-delimited format has

//   10 fixed fields: callsign, year, month, day, hour, min, sec, lat,

//   lon, alt. These are followed by optional fields. This array specifies

//   the optional field list.

//

// Type: enum

// Options:

//     GS

//     TAS

//     TEMP

//     DEW_PT

//     LW

//     ERROR_FLAGS

//     R_BURN

//     L_BURN

//     BURN_IN_PLACE

//     EJECTABLE

//     N_BURN_IN_PLACE

//     N_EJECTABLE

// 1D array - variable length.

//

optional_field_names = {

GS,

BURN_IN_PLACE

};

///////////// burn_in_place_time //////////////////////

//

// Nominal burn time for burn-in-place flares (secs).

// It is assumed that all burn-in-place flares will burn for this length

//   of time. The program will keep track of how many flares are burning

//   on each aircraft, and store that number in SPDB.

// Type: int

//

burn_in_place_time = 240;

//======================================================================

//

// SERIAL TTY input.

//

//======================================================================

///////////// input_device ////////////////////////////

//

// Name of input serial port device.

// Normally /dev/ttyS0 (COM1) or /dev/ttyS1 (COM2). Check that you set

//   the permissions on the serial device to rw-rw-rw-, (666), so that we

//   have read-write permissions for the port.

// Type: string

//

//input_device = "/dev/ttyS1";

///////////// baud_rate ///////////////////////////////

//

// Baud rate.

// Baud rate for incoming serial data port.

//

// Type: enum

// Options:

//     BAUD_300

//     BAUD_1200

//     BAUD_2400

//     BAUD_4800

//     BAUD_9600

//     BAUD_19200

//     BAUD_38400

//

//baud_rate = BAUD_9600;

///////////// dataIs7Bit //////////////////////////////

//

// Flag for 7-bit data.

// Set TRUE for 7-bit data, FALSE for 8-bit data.

// Type: boolean

//

//dataIs7Bit = FALSE;

///////////// twoStopBits /////////////////////////////

//

// Flag for two stop bits.

// If TRUE, 2 stop bits. If FALSE, 1 stop bit.

// Type: boolean

//

//twoStopBits = FALSE;

///////////// enableParity ////////////////////////////

//

// Flag for enabling parity checking.

// If TRUE, parity generation is enabled for outgoing characters, and

//   checking enabled for incoming characters. If FALSE, parity generation

//   and checking is off.

// Type: boolean

//

enableParity = FALSE;

///////////// oddParity ///////////////////////////////

//

// Flag for parity type.

// If TRUE, odd parity. If FALSE, even parity. Only applies if

//   enableParity is TRUE.

// Type: boolean

//

oddParity = FALSE;

//======================================================================

//

// TCP input.

//

//======================================================================

///////////// tcp_server_host_name ////////////////////

//

// Name of TCP server host.

// For connection TCP. The name of the host for the TCP server.

// Type: string

//

tcp_server_host_name = "192.168.1.1";

///////////// tcp_server_port /////////////////////////

//

// TCP server port number.

// For connection TCP. The number of the port for the TCP server.

// Type: int

//

tcp_server_port = 2003;

///////////// send_tcp_handshake //////////////////////

//

// Option to send TCP handshake sequence to the server to trigger the

//   data flow.

// If true, a sequence of bytes will be sent to the server to trigger

//   the data flow. See 'tcp_handshape_bytes'.

// Type: boolean

//

send_tcp_handshake = FALSE;

///////////// tcp_handshake_bytes /////////////////////

//

// List of bytes to be sent to server for handshaking.

// This array of decimal integers is interpreted as a sequence of

//   individual bytes which are to be sent to the server to trigger the

//   data flow.

// Type: int

// 1D array - variable length.

//

tcp_handshake_bytes = {

13,

10

};

//======================================================================

//

// DATA OUTPUT.

//

//======================================================================

///////////// check_callsigns /////////////////////////

//

// Option to check callsigns.

// If TRUE, only aircraft in the callsigns array are stored. If FALSE,

//   data from all aircraft is stored.

// Type: boolean

//

check_callsigns = FALSE;

///////////// callsigns ///////////////////////////////

//

// Callsign array.

// Array of valid callsigns.

// Type: string

// 1D array - variable length.

//

callsigns = {

"N37360" };

///////////// echo ////////////////////////////////////

//

// Option to echo incoming data string.

// If TRUE, the incoming strings are echoed to stdout.

// Type: boolean

//

echo = FALSE;

///////////// new_line_on_echo ////////////////////////

//

// Option to add new line to echo output.

// If FALSE, carriage return is used instead of new line.

// Type: boolean

//

new_line_on_echo = TRUE;

///////////// output_url //////////////////////////////

//

// Output SPDB URL.

// This is the path to the output data directory. You can use full URL

//   sytnax and optionally put the data to a remote machine.

// Type: string

//

output_url = "$(RAP_DATA_DIR)/spdb/ac_posn";

///////////// valid_period ////////////////////////////

//

// Valid period (secs).

// Time period over which the aircraft position is considered valid.

// Minimum val: 1

// Type: int

//

valid_period = 600;

///////////// output_ascii ////////////////////////////

//

// Option to output ASCII to separate SPDB data base.

// If TRUE, the incoming data strings will be stored as ASCII in a

//   separate SPDB dir.

// Type: boolean

//

output_ascii = TRUE;

///////////// ascii_url ///////////////////////////////

//

// ASCII SPDB URL.

// Use if 'output_ascii' is TRUE.

// Type: string

//

ascii_url = "$(RAP_DATA_DIR)/spdb/ascii_ac_posn";

///////////// output_interval /////////////////////////

//

// Interval at which the data is written to the data base (secs).

// This prevents extremely rapid updates to the data base.

// Type: int

//

output_interval = 2;

//======================================================================

//

// TEST SETUP.

//

//======================================================================

///////////// test_aircraft ///////////////////////////

//

// Details of aircraft for TEST mode.

// The test aircraft have a starting location, speed (knots) and

//   altitude (ft). They wander randomly around. If they go further that

//   100 km from their starting location they are brought back to the

//   start. right_burn and left_burn indicate whether the burners should

//   be on or off. ejectable_interval indicates the time beteen ejectable

//   flares - set to -1 for no ejectables. burn_in_place_interval

//   indicates the time between starting burn_in_place flares - set to -1

//   for no burn_in_place flares. n_bip_at_a_time indictes the number of

//   burn_in_place flares to start at a time.

//

// Type: struct

//   typedef struct {

//      string callsign;

//      double start_lat;

//      double start_lon;

//      double altitude;

//      double speed;

//      boolean right_burn;

//      boolean left_burn;

//      double ejectable_interval;

//      int burn_in_place_interval;

//      int n_bip_at_a_time;

//   }

//

// 1D array - variable length.

//

test_aircraft = {

{

callsign = "JRA",

start_lat = -27.43,

start_lon = 28.87,

altitude = 9500,

speed = 150,

right_burn = TRUE,

left_burn = TRUE,

ejectable_interval = -1,

burn_in_place_interval = -1,

n_bip_at_a_time = 0

}

,

{

callsign = "JRB",

start_lat = -28.57,

start_lon = 28.86,

altitude = 10500,

speed = 250,

right_burn = FALSE,

left_burn = FALSE,

ejectable_interval = 10,

burn_in_place_interval = -1,

n_bip_at_a_time = 0

}

,

{

callsign = "CAN",

start_lat = -28.19,

start_lon = 27.76,

altitude = 11500,

speed = 150,

right_burn = FALSE,

left_burn = FALSE,

ejectable_interval = -1,

burn_in_place_interval = 60,

n_bip_at_a_time = 2

}

};